README (RedHat)

The Linux redhat pcnet32 driver supports the AMD PCnet PCI network cards.  This driver is tested on the Red Hat Linux release 6.0 with 2.2.5-15 kernel and Caldera 2.2.5 kernel.

For installing driver on Redhat Linux

- Install the AMD PCnet PCI adapter in the PCI slot of your machine and boot the system.

- Boot the system. Check if the pcnet32.o driver module is in the /lib/modules/'uname -r'/net"             directory, where "uname -r" is the current version's module directory.

- Test the module by running the command "insmod pcnet32.o"

- Install the driver module using the command install -m 644 pcnet32.o /lib/modules/'uname -r'/net/

- Run the netcfg program. Use the menu selections to select the interfaces, protocols and other configuration information and activate the adapter.

- Add the line "alias eth0 pcnet32" in the file /etc/conf.modules and save to automatically load the module. eth0 represents the pcnet32 adapter's interface. eth1 would represent another adapter if it is installed.

- Reboot. The interface eth0 is active and networking is enabled.

- To build the new driver copy the source files pcnet32.c and pcnet32.h in a temporary directory and build using the compile-command: gcc -D__KERNEL__ -DMODVERSIONS -DMODULE -I/usr/src/linux/net/inet -Wall -Wstrict-prototypes -O6 -m486 -c pcnet32.c. Copy the pcnet32.o file created in the /lib/modules/'uname -r'/net directory and  run the command insmod pcnet32.o

- To enable "Autonegotiation" edit the /etc/conf.modules file to add the line                            "options pcnet32 options=0" below the "alias eth0 pcnet32" line. The following indicate the various options for enabling the various interfaces. Only ONE of the following lines can be added. Only one interface is activated based on the keyword it represents. If neither option is selected the driver/device defaults to autonegotiation. Each option is selected by the number it represents

options pcnet32 options=0		#(Autonegotiation)

options pcnet32 options=9 	#(MII, 10Mbps, Half duplex)

options pcnet32 options=10	#(MII, 10Mbps, Full duplex)

options pcnet32 options=13	#(MII, 100Mbps, Half duplex)

options pcnet32 options=14	#(MII, 100Mbps, Full duplex)

For example the file would contain the following lines to select 10Mbps, Half duplex.

alias eth0 pcnet32

options pcnet32 options=10

- To enable LED reprogramming, read the LED documentation from AMD.

- For activating LED programming on eth0 interface edit the file /etc/conf.modules.

alias eth0 pcnet32

options pcnet32 options=0, led_program=1

Edit the pcnet32.h file to select what each LED should represent. Go to the structure LED_STRUCT at the bottom and for eth0 go to index 0 and change the values in the 0th array elements to the values required by you. Make sure the syntax is right by checking the example array provided on the top. For eth1 go to the index 1 and so on.

- Recompile and load the module as per step 8. Reboot to enable the changes.

To get further information please check out the websites  http://www.redhat.com  or http://cesdis.gsfc.nasa.gov/linux/misc/modules.html.